ATWATER KENT MFG. CO.

MODEL 20 #7960

CONDENSERS

Detector phone            .002 mfd  # 8241        500 volts

Detector grid           .00025 mfd  # 8112        500 volts

Plate bypass                .3 mfd  # 14902       450 volts

RESISTORS

Grid suppressors         600   ohms # 4949        wire wound

Detector grid leak         2.0 megs # 15892       1 watt Green

Detector bias            450   ohms # 8190        tapped 180-270 ohms

A-f filament               1.0 ohm  # 8303        brown covered

Detector rheostat         20.  ohms # 8310

R-f rheostat              10.  ohms # 4690

TRANSFORMERS

1st a-f primary         1000   ohms # 8060

1st a-f secondary       8000   ohms

2nd a-f primary         1700   ohms # 7661

2nd a-f secondary       3250   ohms

The detector grid bias resistor is adjacent to the detector socket. It is a flat resistor . The plats bypass condenser is adjacent to the 2nd r-f socket. The phone condenser is located between the detector and 1st a-f sockets.